# SCADA IED Simulator

User Manual

Version 2.0

Modbus TCP Server Simulator for Raspberry Pi

# Table of Contents

1. Introduction

2. System Requirements

3. Installation

4. Configuration

5. Web Interface

6. Modbus Register Reference

7. API Reference

8. Troubleshooting

9. Integration Examples

# 1. Introduction

The SCADA IED Simulator is a professional-grade Modbus TCP server designed for testing, development, and training purposes. It provides a complete simulation environment for Intelligent Electronic Devices (IEDs) commonly used in industrial control systems and SCADA networks.

## 1.1 Key Features

- Full Modbus TCP server implementation supporting all standard function codes

- 100 registers per type (Coils, Discrete Inputs, Holding Registers, Input Registers)

- Web-based Human Machine Interface (HMI) for monitoring and control

- Live data simulation with configurable waveform generators

- Custom variable management with engineering unit scaling

- Configuration export and import functionality

- Real-time activity logging and system statistics

## 1.2 Use Cases

This simulator is designed for:

- Testing SCADA software and RTU/PLC integrations

- Developing and debugging Modbus client applications

- Training personnel on industrial communication protocols

- Simulating field devices during system commissioning

- Creating test scenarios for alarm and trending systems

# 2. System Requirements

## 2.1 Hardware Requirements

| Component | Requirement |
| --- | --- |
| Platform | Raspberry Pi 4 (recommended) or any Linux system |
| Memory | 2 GB RAM minimum (4 GB recommended) |
| Storage | 2 GB free disk space |
| Network | Ethernet or WiFi connectivity |

## 2.2 Software Requirements

- Raspberry Pi OS (64-bit recommended) or Ubuntu/Debian Linux

- Python 3.7 or higher

- Flask 3.0.0 or higher

- pymodbus 3.6.0 or higher

# 3. Installation

## 3.1 Quick Installation

Follow these steps to install the SCADA IED Simulator:

### Step 1: Create Project Directory

mkdir -p ~/scada_simulator/templates

cd ~/scada_simulator

### Step 2: Create Virtual Environment

python3 -m venv venv

source venv/bin/activate

### Step 3: Install Dependencies

pip install flask pymodbus

### Step 4: Copy Application Files

Copy the following files to your project directory:

- enhanced_app.py - Main application file

- templates/index.html - Dashboard interface

- templates/simulation.html - Live simulation interface

- templates/variables.html - Custom variables interface

### Step 5: Run the Simulator

python3 enhanced_app.py

## 3.2 Configuring as a System Service

To run the simulator automatically at system startup, create a systemd service file:

sudo nano /etc/systemd/system/scada-simulator.service

Add the following content:

[Unit]

Description=SCADA IED Simulator

After=network.target

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi/scada_simulator

ExecStart=/home/pi/scada_simulator/venv/bin/python3 enhanced_app.py

Restart=always

[Install]

WantedBy=multi-user.target

Enable and start the service:

sudo systemctl daemon-reload

sudo systemctl enable scada-simulator

sudo systemctl start scada-simulator

# 4. Configuration

## 4.1 Network Settings

The default network configuration is defined in enhanced_app.py:

| Parameter | Default | Description |
| --- | --- | --- |
| MODBUS_ADDRESS | 254 | Modbus Unit ID |
| SERVER_IP | 0.0.0.0 | Listen on all interfaces |
| SERVER_PORT | 5002 | Modbus TCP port |
| FLASK_PORT | 5000 | Web interface port |

## 4.2 Firewall Configuration

If using UFW firewall, allow the required ports:

sudo ufw allow 5000/tcp    # Web interface

sudo ufw allow 5002/tcp    # Modbus TCP

# 5. Web Interface

Access the web interface by navigating to http://<device-ip>:5000 in a web browser.

## 5.1 Dashboard

The Dashboard provides real-time monitoring and control:

- System Statistics: Uptime, total requests, and server status

- Connection Information: Protocol, IP address, port, and Unit ID

- Register Controls: Toggle digital values and set analog values

- Activity Log: Real-time display of all register changes

## 5.2 Live Simulation

The Live Simulation page provides four waveform generators:

- Sine Wave: Configurable amplitude, frequency, and offset

- Ramp/Sawtooth: Linear increase from minimum to maximum value

- Random Walk: Simulates noisy sensor readings

- Square Wave: Alternates between high and low values

The Digital Pulse section allows toggling of discrete inputs to simulate events such as breaker trips or alarms.

## 5.3 Custom Variables

Create custom variables with engineering unit scaling:

- Define meaningful names (e.g., VOLTAGE_L1, PRESSURE_TANK1)

- Map to specific register types and addresses

- Configure scaling to convert raw values to engineering units

- Export and import variable configurations

# 6. Modbus Register Reference

## 6.1 Register Types

| Register Type | Function Codes | Address Range | Access |
| --- | --- | --- | --- |
| Coils | 01, 05, 15 | 0-99 | Read/Write |
| Discrete Inputs | 02 | 0-99 | Read Only |
| Holding Registers | 03, 06, 16 | 0-99 | Read/Write |
| Input Registers | 04 | 0-99 | Read Only |

## 6.2 Supported Function Codes

| Code | Function | Description |
| --- | --- | --- |
| 01 | Read Coils | Read digital output status |
| 02 | Read Discrete Inputs | Read digital input status |
| 03 | Read Holding Registers | Read analog output values |
| 04 | Read Input Registers | Read analog input values |
| 05 | Write Single Coil | Write single digital output |
| 06 | Write Single Register | Write single analog value |
| 15 | Write Multiple Coils | Write multiple digital outputs |
| 16 | Write Multiple Registers | Write multiple analog values |

# 7. API Reference

The simulator provides a REST API for programmatic access.

## 7.1 Status Endpoints

### GET /api/status

Returns current values of all registers (first 10 of each type).

### GET /api/system_status

Returns system statistics including uptime, request count, and recent changes.

## 7.2 Register Write Endpoints

### POST /api/set_coil

Request body: {"address": 0, "value": 1}

### POST /api/set_discrete_input

Request body: {"address": 0, "value": 1}

### POST /api/set_holding_register

Request body: {"address": 0, "value": 1000}

### POST /api/set_input_register

Request body: {"address": 0, "value": 1000}

## 7.3 Configuration Endpoints

### GET /api/export_config

Exports complete system configuration as JSON.

### POST /api/import_config

Imports configuration from JSON payload.

### GET /api/custom_variables

Returns all custom variable definitions.

### POST /api/custom_variables

Creates or updates a custom variable.

# 8. Troubleshooting

## 8.1 Web Interface Not Accessible

- Verify the application is running: ps aux | grep enhanced_app

- Check the port is listening: sudo netstat -tlnp | grep 5000

- Verify firewall rules: sudo ufw status

- Test locally: curl http://localhost:5000

## 8.2 Modbus Connection Failed

- Check port 5002 is listening: sudo netstat -tlnp | grep 5002

- Verify firewall allows port 5002: sudo ufw allow 5002/tcp

- Confirm correct Unit ID (254) is configured in your client

- Test connectivity: ping <device-ip>

## 8.3 Service Won't Start

- Check service status: sudo systemctl status scada-simulator

- View logs: sudo journalctl -u scada-simulator -f

- Verify file paths in service file are correct

- Check Python virtual environment is properly configured

## 8.4 Finding Device IP Address

hostname -I

Or for more detail:

ip addr show

# 9. Integration Examples

## 9.1 Python Client Example

from pymodbus.client import ModbusTcpClient

client = ModbusTcpClient('192.168.1.100', port=5002)

client.connect()

# Read 10 input registers starting at address 0

result = client.read_input_registers(0, 10, slave=254)

print(result.registers)

# Write value 1000 to holding register 0

client.write_register(0, 1000, slave=254)

client.close()

## 9.2 SEL RTAC Configuration

Configure the following settings in SEL RTAC:

- Protocol: Modbus TCP

- IP Address: <Raspberry Pi IP>

- Port: 5002

- Unit ID: 254

- Scan Rate: 1000ms (recommended)

## 9.3 Generic SCADA Configuration

For most SCADA systems, configure:

- Device Type: Modbus TCP Slave

- Connection: TCP/IP

- IP Address: <Device IP>

- Port: 5002

- Slave/Unit ID: 254

End of Document