SCADA IED Simulator

Christian Boiser

Department of Electrical Engineering

University of Idaho

Moscow, Idaho

bois7952@vandals.uidaho.edu

Kai Fulfer
Department of Mechanical Engineering
University of Idaho

Moscow, Idaho

fulf2113@vandals.uidaho.edu

Thomas Schmidt 
Department of Computer Engineering
University of Idaho

Moscow, Idaho

schm5321@vandals.uidaho.edu

Abstract— Supervisory Control and Data Acquisition (SCADA) system provides real-time monitoring, fault detection and data acquisition on field devices. They are applied across a wide range of industries from electrical utilities, water distribution, oil farms, and other large-scale infrastructures. To achieve these functions, SCADA systems utilize multiple Intelligent Electronic Devices (IED), including smart relays and meters deployed at various locations throughout the network. These IED’s collects operational data and tracks the status of each location. Thereby playing a critical role in maintaining systems integrity and ensuring the system operates under nominal conditions.

Keywords—SCADA, data acquisition, IED’s, relays, meters.

# Introduction

SCADA plays a critical role in power distribution. It offers real-time monitoring, fault detection, data acquisition, automation, and cybersecurity. SCADA systems network a variety of field devices such as relays, meters, breaker controllers and other Intelligent Electronic devices (IED’s) to manage the power system’s information demands such as data logging, communication, and control and protection, all happening in real time. However, these IEDs come with significant drawbacks, including excessive costs (upwards of tens of thousands of dollars), limited scalability, bulky form factors, and lack of desktop simulation versatility. To cover these challenges, a lot of companies have relied on developing test setups via simulation software to produce data without the need to purchase the hardware. However, such software would still require additional resources such as licensing fees which could reach over $10,000 for yearly licenses, not including additional tailored features requested by the company. Additionally, simulation software like RTDS requires a level of knowledge to be used efficiently, the company would need to invest time to train their engineers to justify the need to get the license. Lastly, a simulator like the RTDS requires an RTDS rack that would be costly on the upfront and adding construction of a room specifically just for these racks and its maintenance. This project aims to address these drawbacks by designing a low-cost alternative to emulate substation IED behavior that is both scalable and versatile to support the need for data generation and testing.

# Methodology

## Determining the Hardware

The team decided to implement Raspberry Pi 4b as the hardware for the simulator. It is the cheapest version of Raspberry Pi that has the peripherals we need for our application.

## Device Configuration

Used Python as the main programming language for the device. Developed a code for simulating analog and digital data and sending it via MODBUS protocol. A webpage designated to display the present data being simulated as well as a gateway for users to configure the PI simulator was created.

## Mechanical Design

## The mechanical design process used an iterative CAD approach to meet the geometric requirements of internal components and compatability with DIN rail. Physical prototypes we 3D printed to validate the form, fit and performance before finalizing the design.

## Testing

The test was done by using a SEL Real-Time Automation Controller (RTAC), to evaluate the capability of our PI IED simulator to send analog and digital signals to a remote terminal unit (RTU)

# Results

The SCADA IED Simulator is a professional-grade Modbus TCP server that implements all standard function codes (01–06, 15, 16) with 500 registers per type across coils, discrete inputs, holding registers, and input registers. The simulator comes pre-configured with over 260 power industry variables covering electrical measurements, protection elements, transformer monitoring, generator data, and more. It features a web-based HMI with a real-time dashboard and live waveform simulation capabilities, along with a REST API for programmatic control and configuration of import/export functionality. Designed to run on Raspberry Pi 4 or any Linux system, the complete project is available on GitHub with easy installation instructions and a comprehensive user guide.

# Conclusion

Over the course of the two semesters, our team developed an IED PI simulator capable of simulating multiple analog and digital signals. To support user interaction, a web interface was also created to serve as the HMI that allows users to configure the data to be simulated by the PI simulator.

Additionally, a user manual guide was created so that new users can follow to configure the device. Finally, a finalized PI housing design that is compatible with DIN rail which is suitable for laboratory and testing environment.

Our team has faced multiple challenges throughout the course of project development, and we have come to the conclusion that our initial goal was unlikely to be achieved. Despite that, by focusing on the main task of the project we became efficient on where we put our effort given the time constraint and individual constraints we had, to meet our goals the best we could.